PHOTONIC FENCE MONITORING DEVICE

PFMD

USER GUIDE

Copyright © Photonic Sentry. All rights reserved.

Contacting Photonic Sentry:

Photonic Sentry, LLC

14360 Se Eastgate Way Bellevue, WA 98007-6462 USA

For product information, service, and technical support:

support@PhotonicSentry.com

Symbols and terms on the product

The following symbol(s) may appear on the product:

INPUT

DEVICE

MASS

STORAGE

POWER

MONITOR

NETWORK

Page | 1

Revision Log

Page | 2

REV #

DATE

AFFECTED SECTNS

CHANGE DESCRIPTION

AUTHOR/OWNER

N/C

02/12/2021

N/A

Initial Release

Arty Makagon

A

04/08/2021

ALL

- Added General Safety Precautions section

- Added Introduction section

- Added Transport and Storage section

- Added Appendix A: Technical Specifications

- Added Appendix B: Recommended Accessories

- Added Appendix C: License Information

Arty Makagon

Contents

1.

2.

3.

4.

5.

6.

7.

General Safety Precautions	4

Introduction	4

Getting Started	4

Recording Session	7

Data Export and Deletion	9

Transport, Storage and Mounting	10

Troubleshooting	10

Appendix A: Technical Specifications	11

1.

2.

3.

4.

5.

6.

Product Description / Key Functionality	11

Operating Environment	13

Interfaces	13

Portability	13

Packaging	14

Serviceability / Maintainability	14

Appendix B: Recommended Accessories	15

Appendix C: License and Warranty Information	16

Page | 3

1. General Safety Precautions

Please follow these safety precautions to avoid injury and prevent damage to the PFMD.









Use this product only as specified.

Only qualified personnel should perform service.

Use only the power cord specified for this product and certified for the country of use.

The PFMD is grounded through the grounding conductor in the power cord. To avoid electric shock, the grounding conductor must be connected to earth ground.

Do not operate the PFMD with any covers removed. Keep product surfaces clean and dry.





2.

Introduction

a.

Product Intended Use

The Photonic Fence Monitoring Device (PFMD) is an optoelectronic system intended for observing and

analyzing the behavior of insects in flight.

b.  Technical Specifications

Please refer to Appendix A: Technical Specifications

3. Getting Started

Setting up a recording session of in-flight insects using your Photonic Fence Monitoring Device (PFMD) is

as simple as pointing the device at the experimental scene, turning it on and erecting the retroreflective backstop. There are a few considerations to keep in mind:

a.	Your PFMD is designed to operate optimally in one of three range configurations, which you

specified at the time of procurement:

For best results, erect the retroreflective backstop (instructions provided separately) at the

maximal range of your PFMD system.

b.  The active region of your PFMD is a truncated rectangular pyramid with the 3m x 3m base

residing on the backstop. A representative LONG-RANGE configuration is depicted in Figure 1:

Page | 4

Range Configuration

Range from PFMD in which Targets can be Detected

SHORT-RANGE

1.5m – 3m

MID-RANGE

2.5m – 5m

LONG-RANGE

5m – 10m

PFMD

Figure 1: Representative PFMD configuration

Please ensure that the entirety of the experimental scene of interest resides within the active

region. The Scene Preview, described in section 4a, can be used as a guide.

c.

The PFMD was designed to support several mounting configurations:

d.

Interacting with the onboard user interface will require the use of a video monitor, keyboard,

and mouse. Connect a video monitor* to the HDMI port on the rear panel of the PFMD; use the USB hub furnished in the PFMD kit to connect a keyboard* and mouse* to the USB-2 port on the rear panel of the PFMD. Refer to Figure 2 for reference.

Connect the power supply furnished in the PFMD kit to the power port on the rear panel of the PFMD and plug the other end into an AC power outlet. Press and release the power button to turn the PFMD on; the power button will illuminate. Refer to Figure 2 for reference.

e.

Page | 5

Mounting Configuration

Notes

TRIPOD

A heavy-duty tripod* may be used to support the PFMD, which is equipped with a threaded tripod mount on the base.

FLOOR

The PFMD may be placed on the floor if that is the optimal vantage point. The viewing angle can be adjusted using the tilting mechanism on the front face of the device; use the clamping lever to release and secure the tilting mechanism (Figure 3).

CUSTOM

Alternatively, the PFMD may be placed on any sturdy surface large enough to support the four rubberized feet on the base of the device.

POWER BUTTON

POWER PORT

HDMI PORT

USB-2 PORT

USB-3 PORT

NETWORK PORT

Figure 2: PFMD rear panel features

ADJUST KICKSTAND TO ACHIEVE DESIRED TILT ANGLE

ROTATE CLAMPING LEVER TO RELEASE OR SECURE

Figure 3: Tilt mechanism

Page | 6

4.

Recording Session

a.

Once the PFMD is powered on, the user interface (formally, the PFMD Capture Client) is

accessible. The first step is to verify appropriate pointing of the PFMD. The default view of the PFMD user interface (Figure 4) depicts the area visible to each of the two cameras (left and right, in concert comprising the stereoscopic vision system). Use this view to aim the PFMD to optimize the experimental scene visible to the device.

Figure 4: GUI default scene

b.

Adjust illumination levels to match the configuration of the experimental scene. The

illumination adjustment dialog box is accessed using the SETTINGS button. The goal is to manipulate the Brightness slider for each camera until the portion of interest within each image

(left and right) is adequately bright without being blown out.

See Figure 5 for reference.

Figure 5: Brightness adjustment

Page | 7

c.

A specified Region of Interest can be used to exclude areas of the scene which are not germane

to the experiment. Setting the Region of Interest is accomplished using the ROI toggle and subsequently adjusting the green rectangular ROI box for each camera independently.

Eliminating the Region of Interest is accomplished using the CLEAR ROI button. See Figure 6 for reference.

Figure 6: Region of Interest selection

d.

Once the product is oriented appropriately and illumination and ROI are optimized, attention is

turned to the RUN TIMED, START, and STOP buttons under the camera images. Selecting the RUN TIMED option allows the user to run the recording for a predetermined segment of time (specified in units of minutes). Conversely, if the experiment duration is not fixed, the user is able to initiate the recording using the START button and terminate it manually at a later time using the STOP button. See Figure 7 for reference.

A

B

Figure 7: Capture options: A: RUN TIMED; B: RUN MANUAL

Page | 8

5. Data Export and Deletion

Once a recording is terminated it appears in the list on the left portion of the window. Exporting a data

file is accomplished as follows:

Insert a USB-3 capable external mass storage device* into the USB-3 port (Figure 2). Select the data file to be exported from the list on the left portion of the window. Click the EXPORT SELECTED button (Figure 8).

Select the external mass storage device form the Please Select output location dialog box

a.

b.

c.

d.

Figure 8: Data file export

A data file that is no longer necessary can be deleted by selecting the DELETE SELECTED button and

confirming the action in the ensuing dialog box (Figure 9)

Figure 9: Data file deletion

Page | 9

6.

Transport, Storage and Mounting

a.

The PFMD is delivered in packaging which was designed to keep it safe during transport and

storage. Please retain this original packaging and use it to transport and store the PFMD when it is not being utilized for data collection.

When carrying the PFMD outside of its packaging, please support it using both handles on the lid of the device. The handles can be folded flat against the lid when not in use.

b.

7.

Troubleshooting

For assistance, please contact support@PhotonicSentry.com.

Page | 10

Appendix A: Technical Specifications

1.

Product Description / Key Functionality

Page | 11

Illumination

Onboard IR illumination to allow day or night operation

Run Time

1-week continuous operation, including adequate onboard data storage (upload to web app for post-processing when network is available)

Active Volume

NOTE: for pictorial descriptions, please see Figure A10

- 3m x 3m x RANGE square pyramid frustum, where RANGE is set at the time of unit production as specified by the customer, varying from 3m to 10m

- For example:

  - Configuration A (largest, aka LAB): 3m X 3m X 10m frustum, blind to targets at range less than 5m; see Figure A10A

  - Configuration B (smallest, aka MINI): 3m X 3m X 3m frustum, blind to targets at range less than 1m; see Figure A10B

Target Capacity

- Ability to track no fewer than 1,000 non-intersecting targets concurrently

- Ability to track no fewer than 100 multiple intersecting targets (e.g., swarms)

Resolution

- Reliably tracks targets with characteristic length of 4mm or greater (e.g., mosquitoes)

- Smaller targets are likely to be detected

Recorded Data

- 3-D position of each unique target, maintaining track continuity across specified active region(s)

  - trajectory uncertainty, with respect to other targets, of ±2mm along height and width axes and ±5mm along the range axis

- Apparent size estimate of each unique target

Figure A10: Reference Photonic Fence Monitoring Device Installations: A – LAB configuratio n, B – MINI configuration

Page | 12

y, HEIGHT

B

z, RANGE	x, WIDTH

y, HEIGHT

A

z, RANGE	x, WIDTH

2. Operating Environment

*IP54 Rated

3.

Interfaces

4.

Portability

Page | 13

Initial Setup and Calibration Routine

Accomplished within 4 hours

Teardown

Accomplished within 2 hours

Mass

7kg, capable of being moved and hoisted onto tripod by one person

Hardware

Electrical Power

- 48VDC input; 110V - 240 V adapter included in kit

Network

- RJ-45 ethernet port

Human Interface Devices

- 1 x HDMI port for video

- 1 x USB-2 port for peripherals; USB-2 hub included in kit

External Storage

- 1 x USB-3 port

Software

UX

- Onboard: minimal, intended for data-gathering setup and troubleshooting

- Web app: includes pre-configured dashboards, visualization tools and data export features

File Size

- Small, capable of being transferred using a portable USB mass storage device

Data Processing

- Near-real-time visualization of key parameters (e.g., # of targets tacked, recording duration, etc.)

- Additional processing accomplished post-data-collection

Power Supply Input

100 – 240 VAC, ± 10%, 50/60 Hz

Power Consumption

60W, maximum

Ambient Temperature

- Operational: 5°C - 45°C

- Storage: -20°C - 60°C

Ambient Humidity

0 – 90% RH

Sun Exposure

Direct sunlight may raise the operating temperature of the optoelectronic hardware beyond acceptable limits and may also impose excessive optical noise. Installations which would normally expose the PFMD to direct sunlight will need to erect a sunshade.

Moisture Resistance*

Indoor/outdoor, including capability to sustain high humidity and rain

Dust Protected*

Protected from limited dust ingress

5.

Packaging

6.

Serviceability / Maintainability

Page | 14

Required Maintenance

- Wipe clean retroreflective backdrop, on demand

- Wipe clean front optical surface of the PFMD, on demand

Customer Support System

- Email / phone

- Remote troubleshooting capability via public network

Returns Process

See Appendix C: License and Warranty Information

Operability Self-Test

To be executed by customer at customer site.

In the Box

- Photonic Fence Monitoring Device (PFMD)

- Power supply

- USB-2 hub

- Retroreflective screen

- Free-standing frame for mounting retroreflective screen

Shipping Crate Impact Requirements

Withstands drop from 1m height when encased in OEM packaging

Appendix B: Recommended Accessories

Page | 15

ITEM

RECOMMENDED MODEL

Tripod

Topcon 60810 Extra Heavy-Duty Aluminum Elevating Tripod

Tilt Plate

Spectra Precision M401 Tilting Base Adaptor

External USB-3 Drive

Western Digital Elements 1TB Portable External Hard Drive WDBUZG0010BBK-WESN

Retroreflective Fabric

3M Scotchlite Reflective Material 8910 Silver Fabric

Touchscreen Monitor

Corkea 14 Inch Touchscreen Display T140A

Appendix C: License and Warranty Information

The Photonic Fence Monitoring Device is provided in accordance with the terms available on the device

and as may be updated from time to time. Those terms can also be found on the PFMD.net website. At the time of last revision of this user manual, the license terms applicable to the device and associated web app were as follows:

Photonic Sentry

Terms of Purchase and Service

Last Updated: March 2, 2021

PLEASE READ THESE TERMS BEFORE USING THIS HARDWARE, THE RELATED SITE AT PFMD.NET OR

USING SERVICES PROVIDED BY PHOTONIC SENTRY.

These Terms of Purchase and Service (“Terms”) apply to your purchase or use of hardware products,

including the Photonic Fence Monitoring Device, retroreflector, and retroreflector stand (collectively, “Hardware”) and services, software, platform, data analysis tools, visualizations, and websites, including photonicsentry.com and PFMD.net (collectively, “Services”) from Photonic Sentry, LLC (“Photonic Sentry”, “we”, or “our”). These Terms are a legally binding contract between you and Photonic Sentry regarding your purchase of any Hardware and your use of any Services.

BY CLICKING ON THE “SUBMIT” OR OTHER SIMILAR BUTTON OR BY CLICKING “I ACCEPT,” CREATING AN

ACCOUNT, OR BY DOWNLOADING OR INSTALLING SOFTWARE OR OTHERWISE ACCESSING OR USING THE SERVICES, OR BY PURCHASING HARDWARE FROM US YOU AGREE THAT YOU HAVE READ AND UNDERSTOOD, AND, AS A CONDITION TO YOUR PURCHASES AND USE OF THE SERVICES, YOU AGREE TO BE BOUND, BY THESE TERMS AS WELL AS THE PHOTONIC SENTRY PRIVACY POLICY (also available for

review on the websites identified as part of the Services). If you are purchasing or using our Hardware or Services as the employee or agent of an organization, you are agreeing to these Terms and the Privacy Policy on behalf of that organization. If you do not agree, do not purchase any Hardware, and you do not have our permission to use any Services.

If you are making purchases or accessing and using the Services from the European Union, certain

separate terms may apply to you as indicated below.

Photonic Sentry Hardware and Services Overview; Eligibility

Photonic Sentry’s Hardware and Services are intended to be used in the study and eradication of flying

insects. If you are an entity, organization, or company, the individual accepting these Terms on your behalf represents and warrants that they have authority to bind you to these Terms and you agree to be bound by these Terms.

Accounts

Accounts and Registration. To purchase Hardware, and to access most features of the Services, you

must register for an account. When you register for an account, you may be required to provide us with some information about yourself, such as your name, email address, or other contact information. You

Page | 16

agree that the information you provide to us is accurate and that you will keep it accurate and up-to-

date at all times. When you register, you will also be asked to provide a password. You are solely responsible for maintaining the confidentiality of your account and password.

User Accounts. Each user must have their own account, and you agree that you will not allow others to

use your account. If any person, other than yourself, uses your account, you will be in violation of these Terms and Photonic Sentry may take such measures as we deem necessary in our discretion, including termination of your account with us and ceasing to provide you with access to the Services. If you believe that your account is no longer secure, then you must immediately notify us at customers@photonicsentry.com.

Use of Hardware and Services. You represent that Hardware you purchase is for your own use and not

for resale. Use of Hardware and Services may be subject to local laws, regulations, and ordinances in your jurisdiction. You are responsible for complying with all applicable laws. It is your responsibility to determine whether your use of the Hardware and Services complies with local laws, regulations, and ordinances. You may request information about the Hardware and Services from Photonic Sentry to assist you in making your determination, but Photonic Sentry may not have such information available and will not be liable for errors in that information or for your determination. If you do not have sufficient information to determine whether your use of the Hardware or Services will comply with all applicable laws, regulations, and ordinances, then you should not purchase the Hardware or use the Services. You must also use each item of Hardware in strict accordance with any applicable user documentation. Use of the Services is subject to these Terms. If you violate the Terms, your access to Services may be suspended or terminated, and you may not be able to use the data collected by the Hardware, or you may lose Customer Data (defined below). Photonic Sentry will not be liable for your inability to use any Hardware or Service or loss of any Customer Data.

Purchases of Hardware and Subscription Services

Ordering Generally; Deposits. All orders for Hardware are subject to Photonic Sentry’s acceptance.

Except to the extent prohibited by applicable law, Photonic Sentry may accept, decline, or place limits on your order for any reason. You do, however, acknowledge that by clicking on the “Buy”, “Buy Now”, “Purchase”, “Pay Now”, “I Accept” or other similar button, or by paying a deposit to purchase Hardware, you enter into an obligation to pay for Hardware or the applicable Service. Deposits paid for Hardware are nonrefundable.

Availability. Purchases of Hardware are subject to estimated availability as described on the Photonic

Sentry website at the time that you place your order or as otherwise communicated to you by Photonic Sentry. Any availability or shipping date for an accepted order is an estimate only, and the actual shipping date for any accepted order will depend on a variety of factors including manufacturing schedule, the date of your order, and the date your payment was received in full by Photonic Sentry.

Pricing. Photonic Sentry reserves the right to determine pricing for Hardware and Services, as well as

charges for shipping Hardware. Photonic Sentry will make reasonable efforts to keep pricing information published on the website up to date. Photonic Sentry, at its sole discretion, may make promotional offers with different features and different pricing to any of Photonic Sentry’s customers. These promotional offers, unless made to you, will not apply to your offer or these Terms. Except in the case of sales tax for certain locations which may be identified when payment is taken, all import duties, taxes,

Page | 17

tariffs, and other charges are not included in the Hardware or Services price or shipping costs. These

charges are your responsibility. Please check with your state and country’s customs office to determine if there will be additional costs prior to completing your order. All orders for Hardware must be paid in full, including shipping, prior to shipment by Photonic Sentry of Hardware to you. Photonic Sentry reserves the right to suspend shipment and performance until full payment is received.

Invoice Terms. You may request invoice terms to pay for Hardware or Services by contacting Photonic

Sentry at info@photonicsentry.com. Photonic Sentry may grant invoice terms in its sole discretion. If granted, the invoice will state the purchase price for all Hardware or Services (as applicable) plus any freight, taxes, insurance or other applicable costs, and you will pay all invoiced amounts by electronic fund transfer to an account identified by Photonic sentry or by sending a check directly to Photonic Sentry within the time specified in the invoice terms. All payments by you will be made in U.S. dollars.

Shipping. All Hardware will be packed for shipment in Photonic Sentry’s standard containers, marked for

shipment to your address set forth above or to an address specified in the Purchase Order, and delivered to a carrier or forwarding agent chosen by you. Should you fail to designate a carrier or forwarding agent, Photonic Sentry will make such designation. Shipment will be F.O.B. manufacturer’s facility as specified in the invoice, (the “Delivery Point”), at which time risk of loss and title will pass to you. All freight, insurance and other shipping expenses from the Delivery Point, as well as any special packing expenses requested by you, will be borne by you. Photonic Sentry reserves the right in its sole discretion to allocate inventories and current production when it determines such allocation is necessary.

Export Control. You acknowledge that Hardware and Services may be subject to export control laws and

other laws and regulations of the United States and other countries, and that if Photonic Sentry ships Hardware to you, Hardware may be impounded or otherwise confiscated by customs or other authorities. You are responsible for compliance with all applicable export control laws and regulations. You represent that you will not export, re-export, or transfer indirectly or directly any Hardware, or use any Service, outside of the United States without obtaining proper authorization from the applicable government agencies. Without limiting the immediately preceding sentence, you will not export, re-export, or transfer directly or indirectly any Hardware to: (i) an embargoed/terrorist supporting country, including Cuba, Iran, North Korea, Syria, Sudan, or any other such country as determined by the U.S. government; (ii) a person or entity barred by the U.S. Government on export activity lists, including persons or entities on the Treasury Department Specially Designated National List, Entities List, and Denied Persons List; or (iii) any destination for an end use that is prohibited by applicable law. You will defend and hold Photonic Sentry harmless against all claims, damages, or liability resulting from breach of the foregoing.

Inspection. You should carefully examine all deliveries of Hardware upon delivery, and notify Photonic

Sentry of any alleged error, shortage, defect or non-conformity of Hardware within 5 days after receipt. Your failure to examine and report will constitute a waiver of any claim against Photonic Sentry arising under these Terms or by law with respect to any such error, shortage, defect, or non-conformity reasonably discoverable by examination. You agree to make any and all claims for damage or loss in transit against the carrier.

Subscription Service. Use of the Services may require automatically recurring payments for periodic

charges (“Subscription Services”). Photonic Sentry may offer a variety of Subscription Services, including

Page | 18

special promotional plans, which may have differing conditions and limitations, which will be disclosed

to you at sign-up or in other communications made available to you. If you decide to activate a Subscription Service, and unless Photonic Sentry approves invoice terms for you, you authorize Photonic Sentry to periodically charge, on a going-forward basis and until cancellation of either the recurring payments or your account, all accrued sums on or before the payment due date for the accrued sums. The “Subscription Billing Date” is the date when you purchase your first subscription to the Service.

Your account will be charged automatically on the Subscription Billing Date all applicable fees for the next subscription period. The subscription will continue unless and until you cancel your subscription or we terminate it. You must cancel your subscription before it renews in order to avoid billing of the next periodic subscription fee to your account. We will bill the periodic subscription fee to the payment method you provide to us during registration (or to a different payment method if you change your payment information). To ensure uninterrupted service and to enable you to conveniently add additional user accounts, Photonic Sentry or its billing providers (which may not be affiliated with Photonic Sentry) may store your payment details on file. If Photonic Sentry is unable to charge your designated payment method for any reason, Photonic Sentry reserves the right to suspend your paid membership until we receive payment and a working designated payment method for future payments. Please note that it is your responsibility to maintain current credit card or alternate billing information on file with Photonic Sentry or its billing service provider. Photonic Sentry may increase the subscription fee for any subsequent subscription period if Photonic Sentry gives you advance notice of the increase before it applies. Such notice will be sent to the email address which you provide to Photonic Sentry.

You may cancel a Subscription Service by contacting us at: customers@photonicsentry.com. Payments for Subscription Services are nonrefundable. If you cancel, change your Subscription Service, or if your account is otherwise terminated under these Terms, you will not receive a credit or refund, including for partially unused periods of a Subscription Service.

Trial Periods. From time to time, Photonic Sentry may offer trials of Subscription Services for a specified

period without payment or at a reduced rate (a “Trial”). Photonic Sentry may determine your eligibility for a Trial and withdraw or modify a Trial at any time without prior notice and with no liability, to the extent permitted under applicable law. For some Trials, Photonic Sentry may require you to provide your payment details to start the Trial. By providing such details you agree that Photonic Sentry may automatically begin charging you for the Subscription Service on the first day following the end of the Trial and thereafter in accordance with the paragraph above. IF YOU DO NOT WANT THIS CHARGE, YOU MUST CANCEL THE APPLICABLE SUBSCRIPTION SERVICE BEFORE THE END OF THE TRIAL WITHIN YOUR

ACCOUNT OR BY CONTACTING PHOTONIC SENTRY AT customers@photonicsentry.com.

Delinquent Accounts. Photonic Sentry may suspend or terminate access to the Service for any account

for which any amount is due but unpaid. In addition to the amount due for the Service, a delinquent account will be charged with fees or charges that are incidental to any chargeback or collection of any unpaid amount, including collection fees.

Support. You may request support for the Service, including any problems with Hardware, by contacting

us at customers@photonicsentry.com. Photonic Sentry is under no obligation to provide specific levels of support, or to meet specific response or resolution times.

Licenses

Page | 19

Hosted Services. “Hosted Services” means the Services provided to you that are not embedded in the

Hardware. Photonic Sentry grants you a personal, non-exclusive, non-transferable, non-sublicensable, revocable license to access and use the Hosted Services for your own personal or your business’s or organization’s use in accordance with all applicable documentation and these Terms.

Embedded Software. “Embedded Software” means software or firmware that is embedded in the

Hardware. Photonic Sentry grants you a non-exclusive, non-sublicensable, revocable license to use Embedded Software solely as embedded on the Hardware.

Third Party Products. To the extent that the Hardware or Services includes or is accompanied by third-

party software or other products (e.g., cloud hosting instances or data analysis tools) that Photonic Sentry provides to you or that is otherwise identified in the Documentation as being required to use properly such Hardware or Services (“Third-Party Products”), the Third-Party Products and their use by you are subject to all license and other terms that accompany such Third-Party Products. You will abide by and comply with all such terms.

Documentation. To the extent that Photonic Sentry makes available any user manuals, help files,

specification sheets, or other documentation, in any form, relating to the Hardware or Services (“Documentation”), Photonic Sentry hereby grants to you a non-exclusive, non-sublicensable right and license under Photonic Sentry’s rights in the Documentation to use such Documentation solely to enable you to exercise your rights under the applicable license or grant of access and usage rights for the Hardware or Services as set forth in these Terms.

Prohibited Use. Except as otherwise explicitly provided in these Terms or as may be expressly permitted

by applicable law, you will not, and will not permit or authorize third parties to: use the Hardware or Services in connection with the development of a competitive product; (b) circumvent or disable any security or other technological features or measures of the Hardware or Services or use the Hardware or Services in a manner that you reasonably believe poses a threat to the security of Photonic-Sentry-controlled computer systems; (c) modify, translate, reverse engineer, decompile, disassemble, or otherwise derive the source code or the underlying ideas, algorithms, structure, or organization from the Hardware or Services (except to the extent that applicable law prevents the prohibition of such activities); (d) use or access any Hardware or Service in a manner that materially impacts or burdens Photonic Sentry or Photonic Sentry’s servers and other computer systems; (e) use the Services for any illegal purpose or in violation of any applicable local, state, national, or international law; (f) violate any right of a third party, including by infringing or misappropriating any third party intellectual property right; or (g) attempt to interact with the Services in a way not authorized by these Terms.

Notice to Government End-Users. Photonic Sentry provides the Hosted Services and Embedded

Software, including related data and technology, for ultimate federal government end use solely in accordance with the following: the government hereby agrees that the software qualifies as “commercial” computer software. Government technical data and software rights related to the software include only those rights customarily provided to the public as defined in these Terms. This customary commercial license is provided in accordance with FAR 12.211 (Technical Data) and FAR

12.212 (Software) and, for Department of Defense transactions, DFAR 252.277-7015 (Technical Data – Commercial Items) and DFAR 227.7202-3 (Rights in Commercial Computer Software or Computer Software Documentation). If a government agency has a need for rights not granted under these Terms, it must negotiate with Photonic Sentry to determine if there are acceptable terms for granting those

Page | 20

rights, and a mutually acceptable written addendum specifically granting those rights must be included

in any applicable agreement.

Ownership

Ownership; Proprietary Rights. The Hardware, Embedded Software, and the visual interfaces, graphics,

design, compilation, information, data, computer code (including source code or object code), software, services, and all other elements of the Services are protected by intellectual property and other laws, and are the property of Photonic Sentry or its third-party licensors. None of the Hardware or Services or the contents thereof may be copied, reproduced, republished, downloaded, uploaded, posted, displayed, transmitted or distributed in any way except through the proper use of the Hardware and Services and nothing in the Services or on the Photonic Sentry websites shall be construed to confer any license under any of Photonic Sentry or its licensors or affiliates’ intellectual property rights, whether by estoppel, implication or otherwise, except as explicitly permitted herein. Nothing in these Terms will be construed as restricting or preventing Photonic Sentry or its licensors from exploiting any of its or their intellectual property rights, without any obligation to you.

Customer Data. As between Photonic Sentry and you, you retain all right, title, and interest, including all

intellectual property rights, in and to: (a) the specific experimental data that you upload or input into the Services; and (b) data that is generated and made available to you by any Hardware or Software through use of the data described in part (a) above ((a) and (b) collectively, “Customer Data”). You hereby grant Photonic Sentry a non-exclusive, worldwide, royalty-free, fully paid, sublicensable, fully transferable, irrevocable license to use, process, transmit, store, and disclose the Customer Data: (i) for the purpose of exercising Photonic Sentry’s rights and performing its obligations under these Terms; and

(ii) in perpetuity, for its business and research and development purposes, including to develop and improve Photonic Sentry’s products and services.

Privacy Policy; Use, Transmission and Disclosure of Private Information

A complete statement of Photonic Sentry's current Privacy Policy can be found on the PFMD.net and

photonicsentry.com websites. Photonic Sentry's Privacy Policy is expressly incorporated into these Terms by reference.

In connection with the use of certain Services, you may be asked to provide personal information. You

are advised and should understand that Photonic Sentry may need to transmit this information to a third party in order to provide the Services, and you therefore grant Photonic Sentry a worldwide, royalty-free, nonexclusive, and fully sublicensable license to use, distribute, reproduce, modify, publish and translate this personal information solely for the purpose of enabling your use of the Services.

Because internet transmissions are never completely private or secure, in using the Services you acknowledge that any message or information you send via the Services may be read or intercepted by others, even if there is a special notice that a particular transmission (for example, credit card information) is encrypted. We strive to prevent loss and theft of data, but we cannot guarantee it and will not be liable for it in any way.

You grant permission for Photonic Sentry to use your private information including information and data

provided in the course of accessing and using the Services and information you provide in the account setup and management processes in connection with providing Services to you. You acknowledge and

Page | 21

agree that Photonic Sentry may preserve this information as well as disclose it (including your identity) if

required to do so by law or in the good faith belief that such preservation or disclosure is reasonably necessary to accomplish any of the following: (1) to comply with legal process, applicable laws or government requests; (2) to enforce these Terms; (3) to respond to claims that any content violates the rights of third parties, is offensive or intended to accomplish an illegal purpose; (4) to protect the rights, property, or personal safety of Photonic Sentry, its users, other individuals, or the public; (5) in connection with any investigation or complaint regarding your use of the Services; (6) to identify, contact or bring legal action against someone who may be causing injury to or interference (either intentionally or unintentionally) with Photonic Sentry’s rights or property or the rights or property of visitors to or users of the Services, including Photonic Sentry’s customers.

Term and Termination; Additional Services Terms

Term, Termination of Use; Discontinuation and Modification of the Service. You may terminate your

account at any time by contacting customer service at customers@photonicsentry.com. If you violate any provision of these Terms, your permission from us to use the Services will terminate automatically. In addition, Photonic Sentry may in its sole discretion suspend or terminate your access to the Services, or portions of the Services, during an investigation of a violation of these Terms. Photonic Sentry’s determination that a violation has occurred will be in its sole and absolute discretion. Upon Photonic Sentry’s determination that a violation has occurred, Photonic Sentry may immediately terminate your access to the Service or portions of the Service. Upon termination of these Terms: (a) your license rights will terminate and you must immediately cease all use of the Services; (b) you will no longer have access to your account; (c) you must pay Photonic Sentry any unpaid amount that was due prior to termination; and (d) all payment obligations accrued prior to termination will survive. Photonic Sentry also reserves the right to modify or discontinue the Services at any time (including by limiting or discontinuing certain features of the Services), temporarily or permanently, without notice to you. In the event that Photonic Sentry terminates the Services without cause, Photonic Sentry’s maximum liability will be limited to refund of any amounts actually paid in advance for periods of Service not yet provided, and in the event of complete termination of the Services without cause, for a pro-rated portion of the price of hardware actually paid within the prior 12 months. Photonic Sentry will have no other liability whatsoever on account of any change to the Services or any suspension or termination of your access to or use of the Services.

Additional Service Terms. Your use of the Services is subject to all additional terms, policies, rules, or

guidelines applicable to the Service or certain features of the Services that Photonic Sentry may post on or link to from the Services (the “Additional Service Terms”), such as end-user license agreements for any downloadable software applications, user manuals, or rules, instructions, or directives that apply to a particular feature or content on the Services. All Additional Service Terms are incorporated by this reference into, and made a part of, these Terms.

Hardware Warranty

Photonic Sentry warrants that unmodified Hardware will be free from defects in materials and

workmanship for one year after delivery to the Delivery Point (“Hardware Warranty”). Any modification to the Hardware voids the Hardware Warranty. Photonic Sentry may, in its sole discretion, extend the Hardware Warranty. To be eligible for relief under the Hardware Warranty, you must request return material authorization by contacting Photonic Sentry at customers@photonicsentry.com, and follow the

Page | 22

procedures to return the device as specified by Photonic Sentry. Return Hardware must be shipped at

customer expense. Upon determining whether the Hardware is defective, Photonic Sentry may reimburse for shipping cost of Hardware returned for service during the Hardware Warranty. Photonic Sentry will, at its option and expense, repair or replace any Hardware or replaceable sub-assembly thereof that does not conform to the Hardware Warranty, subject to your notification of Photonic Sentry that such Hardware or sub-assembly does not comply with the warranties detailed in this paragraph, and subject to your compliance with the warranty return procedure detailed in these Terms. Photonic Sentry may use new or reconditioned replacement parts or Hardware units. Repaired or replaced Hardware shall be warranted free of defects in materials and workmanship for the remainder of the original Hardware Warranty pertaining to the originally purchased Hardware or for thirty (30) days from the date of return of the repaired or replaced Hardware to the customer, whichever is longer. This paragraph states your sole and exclusive remedy, and Photonic Sentry’s sole and exclusive liability, for failure of Hardware to conform to the Hardware Warranty.

Disclaimers

PHOTONIC SENTRY DOES NOT PROMISE THAT THE HARDWARE, SERVICES OR ANY CONTENT, ELEMENT

OR FEATURE OF THE SERVICES WILL BE ERROR-FREE OR UNINTERRUPTED, OR THAT ANY DEFECTS WILL BE CORRECTED, OR THAT YOUR USE OF THE HARDWARE OR SERVICES WILL PROVIDE SPECIFIC RESULTS. YOUR USE OF THE HARDWARE, SERVICES, OR ITEMS OBTAINED THROUGH THE HARDWARE OR SERVICES (INCLUDING WITHOUT LIMITATION CUSTOMER DATA) IS AT YOUR OWN RISK. THE HARDWARE, SERVICES, AND CUSTOMER DATA AND OTHER CONTENT CONTAINED THEREIN OR UTILIZED THEREIN OR UTILIZED AS A PART THEREOF ARE DELIVERED ON AN “AS-IS” AND “AS-AVAILABLE” BASIS WITHOUT ANY WARRANTIES OF ANY KIND EXCEPT AS EXPLICITLY SET FORTH IN THESE TERMS, EITHER EXPRESS OR IMPLIED. PHOTONIC SENTRY CANNOT ENSURE THAT ANY FILES OR OTHER DATA YOU UPLOAD TO OR DOWNLOAD FROM THE HARDWARE AND SERVICES WILL BE FREE OF VIRUSES OR CONTAMINATION OR DESTRUCTIVE FEATURES. YOU ARE SOLELY RESPONSIBLE FOR BACKING UP CUSTOMER DATA. PHOTONIC SENTRY IS NOT LIABLE FOR ANY DATA LOSS.

TO THE FULLEST EXTENT PROVIDED BY LAW, EXCEPT FOR THE WARRANTIES EXPLICITLY SET FORTH IN

SECTION 8, PHOTONIC SENTRY DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING ANY IMPLIED WARRANTIES OF ACCURACY, NON-INFRINGEMENT, MERCHANTABILITY, AND FITNESS FOR A PARTICULAR PURPOSE. PHOTONIC SENTRY DISCLAIMS ANY AND ALL LIABILITY FOR THE ACTS, OMISSIONS AND CONDUCT OF ANY THIRD PARTIES IN CONNECTION WITH OR RELATED TO YOUR USE OF ANY PHOTONIC SENTRY HARDWARE OR SERVICES. YOU ASSUME TOTAL RESPONSIBILITY FOR YOUR USE OF THE HARDWARE AND SERVICES. EXCEPT AS EXPRESSLY SET FORTH IN SECTION 8, YOUR SOLE REMEDY AGAINST PHOTONIC SENTRY AND ITS AFFILIATES FOR DISSATISFACTION WITH THE HARDWARE AND SERVICES OR ANY DATA, DOCUMENT, ELEMENT OR CONTENT FOUND THEREIN IS TO STOP USING THE HARDWARE AND SERVICES OR ANY SUCH CONTENT, DATA, DOCUMENT OR ELEMENT. THIS LIMITATION OF REMEDIES IS A PART OF THE BARGAIN BETWEEN THE PARTIES.

The above disclaimer applies to any damages, liability or injuries caused by any failure of performance,

error, omission, interruption, deletion, defect, delay in operation or transmission, computer virus, communication line failure, theft or destruction of or unauthorized access to, alteration of, or use of the Hardware and Services, whether for breach of contract, tort, negligence or any other cause of action.

Page | 23

Photonic Sentry is not an insurer and provides no insurance to users of the Hardware or Services against

any sort of loss.

Limitation of Liability

EXCEPT WHERE PROHIBITED BY LAW, IN NO EVENT WILL PHOTONIC SENTRY BE LIABLE TO YOU FOR ANY

INDIRECT, CONSEQUENTIAL, EXEMPLARY, INCIDENTAL OR PUNITIVE DAMAGES, INCLUDING LOST PROFITS, EVEN IF PHOTONIC SENTRY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

IF, NOTWITHSTANDING THE OTHER PROVISIONS OF THESE TERMS OF PURCHASE AND SERVICE,

PHOTONIC SENTRY IS FOUND TO BE LIABLE TO YOU FOR ANY DAMAGE OR LOSS WHICH ARISES OUT OF OR IS IN ANY WAY CONNECTED WITH YOUR USE OF THE SERVICES OR HARDWARE, PHOTONIC SENTRY’S TOTAL LIABILITY SHALL IN NO EVENT EXCEED: (A) WITH RESPECT TO USE OF THE SERVICES, THE TOTAL OF ANY SUBSCRIPTION OR SIMILAR FEES YOU HAVE PAID WITH RESPECT TO ANY SERVICES IN THE TWELVE MONTHS PRIOR TO THE DATE OF THE INITIAL CLAIM MADE AGAINST PHOTONIC SENTRY; AND

(B) WITH RESPECT TO THE HARDWARE, THE PRICE ACTUALLY PAID FOR THE HARDWARE].

Some jurisdictions do not allow limitations of liability, so the foregoing limitation may not apply to you.

Indemnity

You agree to indemnify, defend and hold harmless Photonic Sentry, its officers, directors, members,

shareholders, predecessors, successors in interest, employees, agents, subsidiaries and affiliates and its affiliates’ predecessors’ and successors in interest’s officers, directors, members, shareholders, employees, and agents from any demands, losses, liability, claims, damages, judgments, awards or expenses (including attorneys’ fees), made against Photonic Sentry by any third party due to or arising out of or in connection with these Terms or your use of the Services or Hardware.

Termination of Services and Access Restriction

Photonic Sentry reserves the right in its sole discretion at any time without prior notice to downgrade or

terminate your access to the Services and to the data stored therein for any reason or no reason, including without limitation, any violation of these Terms. Photonic Sentry reserves the right in its sole discretion at any time without prior notice to take appropriate legal action, including without limitation, referral to law enforcement, for any illegal or unauthorized use of the Photonic Sentry websites and Services. Use of the Hardware requires an active administrative account subscription to the Hosted Services. If Photonic Sentry elects to terminate your account, Photonic Sentry will provide you with notice to your registered email address. Photonic Sentry also reserves the right to modify or discontinue, either temporarily or permanently, any part of its Services or to remove any Content without notice. The Services may also be suspended or unavailable at times due to maintenance, errors, or failures of Photonic Sentry or service providers on which Photonic Sentry depends – in other words, the site may be down just when you want to look at the data stored therein. You agree that Photonic Sentry will not be liable to you or to any third party for any modification, suspension, or discontinuance of your membership in the Services or loss of access to the Content or the information stored as part of the Services on either a temporary or permanent basis, except as otherwise provided in section 7 of these terms.

Page | 24

We have the right to fully cooperate with any law enforcement authorities or court order requesting or

directing us to disclose the identity or other information of anyone posting any materials on or through the Photonic Sentry websites. YOU WAIVE AND AGREE TO HOLD PHOTONIC SENTRY HARMLESS FROM ANY CLAIMS RESULTING FROM ANY ACTION TAKEN BY PHOTONIC SENTRY DURING, OR TAKEN AS A CONSEQUENCE OF, INVESTIGATIONS BY PHOTONIC SENTRY OR LAW ENFORCEMENT AUTHORITIES.

Force Majeure

Photonic Sentry will not be liable to you for any delay in delivery of Hardware, your inability to access

the Services, or any other failure in the performance of or delay in performance of these Terms caused by events outside of Photonic Sentry’s reasonable control, such as an act of God, terrorism, war, political insurgence, insurrection, riot, civil unrest, act of civil or military authority, uprising, earthquake, flood, epidemic, pandemic, labor shortage due to illness or illness mitigation, act of government, or any other natural or man-made condition outside of Photonic Sentry’s control.

Links to Other Sites

The Services and the Photonic Sentry websites may contain links to other independent third-party web

sites (“Linked Sites”). These Linked Sites are provided solely as a convenience to our visitors and users. Such Linked Sites are not under Photonic Sentry’s control, and Photonic Sentry is not responsible for and does not endorse the content of Linked Sites, including any information or materials contained on Linked Sites. You will need to make your own independent judgment regarding your interaction with Linked Sites.

Modifications to These Terms

Photonic Sentry reserves the right to change these Terms on a going-forward basis at any time. Please

check these Terms periodically for changes. If a change to these Terms materially modifies your rights or obligations, you are required to accept the modified Terms in order to continue to use the Service.

Material modifications are effective upon your acceptance of the modified Terms. Immaterial modifications are effective upon publication. Disputes arising under these Terms will be resolved in accordance with the version of these Terms that was in effect at the time the dispute arose.

Violation of These Terms

You agree that Photonic Sentry may, in its sole discretion and without prior notice, terminate your

access to the Services and/or block your future access to the Services if we determine that you have violated these Terms or other agreements or guidelines which may be associated with your use of the Services. You also agree that any violation by you of these Terms will constitute an unlawful and unfair business practice, and will cause irreparable harm to Photonic Sentry, for which monetary damages would be inadequate, and you consent to Photonic Sentry obtaining any injunctive or equitable relief that Photonic Sentry deems necessary or appropriate in such circumstances. These remedies are in addition to any other remedies Photonic Sentry may have at law or in equity.

If Photonic Sentry takes any legal action against you as a result of your violation of these Terms,

Photonic Sentry will be entitled to recover from you, and you agree to pay, all reasonable attorneys’ fees and costs of such action, in addition to any other relief granted to Photonic Sentry. You agree that

Page | 25

Photonic Sentry will not be liable to you or to any third party for termination of your access to the

Services as a result of any violation of these Terms.

Governing Law; Dispute Resolution

You agree that all matters relating to your access to or use of the Services and these Terms and any

dispute or claim arising therefrom or related thereto (in each case, including non-contractual disputes or claims), will be governed by the laws of the United States and by the laws of the State of Washington, without giving effect to any choice or conflict of law provision or rule (whether of the State of Washington or any other jurisdiction). You agree to the personal jurisdiction of and venue in the state and federal courts in King County, Washington, and waive any objection to such jurisdiction or venue. In the event of any controversy or dispute between Photonic Sentry and you arising out of or in connection with your use of the Services, the parties shall attempt, promptly and in good faith, to resolve any such dispute. If we are unable to resolve any such dispute within a reasonable time (not to exceed thirty (30) days), then either party shall be free to pursue any right or remedy available to them under applicable law through these designated courts.

Void Where Prohibited

Although the Hardware and Services are accessible worldwide, not all features, products or services

discussed, referenced, provided or offered by Photonic Sentry are available to all persons or in all geographic locations, or appropriate or available for use outside the United States. Photonic Sentry does not make any representation as to the propriety of using the Hardware or Services in any particular country or jurisdiction. Photonic Sentry reserves the right to limit, in its sole discretion, the provision and quantity of any feature, product or service to any person or geographic area. Any offer for any feature, product or service made as part of the Hardware or Services is void where prohibited. If you choose to use the Hardware or access Services from outside the United States, you do so on your own initiative and you are solely responsible for complying with applicable local laws.

Miscellaneous

If any of the provisions of these Terms are held by a court of competent jurisdiction to be void or

unenforceable, such provisions shall be limited or eliminated to the minimum extent necessary and replaced with a valid provision that best embodies the intent of these Terms, consistent with the remainder of these Terms remaining in full force and effect. These Terms and our Privacy Policy constitute the entire agreement between you and Photonic Sentry with regard to your purchase of Hardware and use of the Services, and any and all other written or oral agreements or understandings that may have existed or which you may believe to exist between you and Photonic Sentry with respect to such purchase or use, including all additional or different terms in any document issued by you, including any purchase order, are hereby superseded and cancelled. Photonic Sentry will not accept any counteroffers to these Terms, and all such offers are hereby categorically rejected. Photonic Sentry’s failure to insist on or enforce strict performance of these Terms shall not be construed as a waiver by Photonic Sentry of any provision or any right it has to enforce these Terms, nor shall any course of conduct between Photonic Sentry and you or any other party be deemed to modify any provision of these Terms. These Terms shall not be interpreted or construed to confer any rights or remedies on any third parties.

Page | 26

Feedback and Information

All feedback, comments, requests for technical support, and other communications relating to the

Websites should be directed to customers@photonicsentry.com.

Any feedback you provide to Photonic Sentry regarding the Hardware, Services, or any other matter

(“Feedback”) shall be deemed to be non-confidential. Photonic Sentry shall be free to use Feedback and other information you may provide on an unrestricted basis, including using it as the basis for changes to or the creation of additional products or services, without any compensation to you whatsoever.

Accordingly, you grant to Photonic Sentry an unrestricted, perpetual, irrevocable, non-exclusive, sublicensable, transferable, fully-paid, royalty-free right to exploit the Feedback in any manner and for any purpose.

Notice; Consent to Electronic Communications

By creating an account, ordering Hardware, or using the Services, you consent to receiving certain

electronic communications from us as further described in our Privacy Policy. Please read our Privacy Policy to learn more about our electronic communications practices. You agree that any notices, agreements, disclosures, or other communications that we send to you electronically will satisfy any legal communication requirements, including that those communications be in writing. All notices given by you to us must be given in writing to the physical mailing address set out below.

Contact Information

The Hardware and Services are offered by Photonic Sentry, LLC located at 14360 SE Eastgate Way,

Bellevue, WA 98007. You may contact us by sending correspondence to that address or by emailing us at customers@photonicsentry.com.

The following software may be included in the onboard software embedded in the Photonic Fence

Monitoring Device:

Page | 27

Software

Licensed Under

License Notice

Arrow

Apache 2

Copyright © 2021 Photonic Sentry, LLC and its Licensors

With regard to code elements licensed under the Apache 2 license the following terms apply: Licensed under the Apache License, Version 2.0 (the "License"); you may not use this file except in compliance with the License. You may obtain a copy of the License at http://www.apache.org/licenses/LICENSE-2.0

Unless required by applicable law or agreed to in writing, software distributed under the License is distributed on an "AS IS" BASIS, WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND,

either express or implied. See the License for the specific language governing permissions and limitations under the License.

Boost

Boost License

Permission is hereby granted, free of charge, to any person or organization obtaining a copy of the software and accompanying documentation covered by this license (the "Software") to use, reproduce, display, distribute, execute, and transmit the Software, and to prepare derivative works of the Software, and to permit third-parties to whom the Software is furnished to do so, all subject to the following:

* Sold separately.

Page | 28

With regard to code elements licensed under the Boost license the following terms apply: THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, TITLE AND NON-INFRINGEMENT. IN NO EVENT SHALL THE COPYRIGHT HOLDERS OR ANYONE DISTRIBUTING THE SOFTWARE BE LIABLE FOR ANY DAMAGES OR OTHER LIABILITY, WHETHER IN CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

GTest LibZIP OpenCV

BSD 3-

clause

Copyright © 2021 Photonic Sentry, LLC and its Licensors. Copyright © 2021 Photonic Sentry, LLC and its Licensors.

With regard to code elements licensed under the BSD license the following terms apply: THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT HOLDER OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

Qt5

LGPL (v3)

This software makes use of the Qt5 library made available under the LGPL license. A copy of the LGPL license is available here: https://opensource.org/licenses/LGPL-3.0

CapnProto Nlohmann JSON

Spdlog

MIT

Copyright © 2021 Photonic Sentry, LLC and its licensors.

With regard to code elements licensed under the MIT license the following terms apply: Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

CUDA

NVIDIA EULA

This software contains source code provided by NVIDIA Corporation.